INTRODUCTORY ROBOTICS · UNIT 1: ENGINEERING FOUNDATIONS # What is Engineering? Intentional Design Class 1 of 4 · 74 minutes

| NAME _______________________ | TEAM _______________________ | DATE _______________________ | PERIOD _______________________ |
| --- | --- | --- | --- |

LEARNING OBJECTIVES - Define intentional design and explain what makes it different from trial-and-error. - Identify design decisions visible in a robot photo and explain their likely purpose. - Set up and make your first entry in your engineering notebook.

PART 1  ·  READING & DISCUSSION (15 MIN)

## What Makes a Design Intentional?

Engineering is the practice of solving problems with constraints — limits you must work within like weight, cost, size, or time. Intentional design means every choice in your robot has a reason. When someone asks "why did you build it that way?" you should have an answer. Trial-and-error means trying things until something works, without understanding why. It can get you a functioning robot, but you won't be able to improve it reliably or explain it to a judge.

1. In your own words, what is the difference between intentional design and trial-and-error?

2. Give one example from everyday life (not a robot) where someone used intentional design. What was the constraint they were working within?

PART 2  ·  ROBOT PHOTO ANALYSIS (20 MIN)

## Reading a Robot Like an Engineer

Your teacher will display a photo of a REV-based robot. Examine it carefully before answering.

| Novice Identify 3 design choices you can see. Use the table below. | Intermediate Analyze two robots; write a comparison paragraph in Part 2B. | Proficient Write a design critique using engineering vocabulary; propose one improvement with rationale. |
| --- | --- | --- |

I DO Watch the teacher model engineering thinking on this example robot

Before you analyze your own robot photo, your teacher will think out loud using the example below. Notice how each observation leads to a "why" question, and how every design choice has a trade-off. EXAMPLE ROBOT — three annotated design decisions OBSERVATION A — Battery Placement I notice: The battery is mounted low and centered on the frame — not up high, not off to one side. I ask "why?": A heavy object mounted low lowers the center of gravity, making the robot harder to tip when turning fast or climbing a ramp. Centering it keeps the robot balanced side-to-side. Trade-off: Low mounting uses interior space that could hold wiring or a second mechanism. OBSERVATION B — Wheel Size & Placement I notice: 4-inch diameter wheels at all four corners, evenly spaced. I ask "why?": Larger wheels mean more distance per motor revolution (higher top speed). Four-wheel contact distributes the load and reduces the chance of getting stuck. Equal spacing makes turning predictable. Trade-off: Bigger wheels raise the chassis height (slightly higher center of gravity) and require more torque to accelerate from a stop. OBSERVATION C — Arm Position I notice: The arm is mounted at the front-center of the robot, not to the side. I ask "why?": Centering the arm keeps the robot balanced when the arm extends — a side-mounted arm would shift the center of mass sideways. Front-mounting means the driver just needs to point the robot at the target. Trade-off: The arm now blocks the forward camera's view when fully extended — the camera must be repositioned higher or a rear camera added. WHAT THE COMPLETED TABLE LOOKS LIKE — teacher's example What I observe; Why the designer probably chose this; What might be traded off. A. Battery mounted low and centered; Lowers center of gravity to prevent tipping; centering keeps the robot balanced side-to-side; Uses interior space that could hold wiring or a second mechanism. B. 4" wheels at all four corners, evenly spaced; More distance per motor rotation (faster); four-wheel contact reduces getting stuck; equal spacing makes turning predictable; Raises chassis height slightly; requires more torque to accelerate from a stop. C. Arm mounted at front-center of chassis; Keeps weight balanced when arm extends; driver only needs to point robot at target; Arm blocks forward camera view when extended — camera must be repositioned.

YOU DO Part 2A — Design Decisions Table

Now it's your turn. Using the robot photo your teacher displays, fill in the table below. Describe what you observe, explain the likely reason for that choice, and identify any trade-off it might create.

| What I observe | Why the designer probably chose this | What might be traded off |
| --- | --- | --- |

I DO Watch the teacher model a comparison paragraph using two example robots

A strong comparison paragraph names specific features, explains what each difference reveals about team priorities, and identifies trade-offs. It doesn't just list differences. TWO EXAMPLE ROBOTS Robot A — Speed-focused; Robot B — Capability-focused. - 6" diameter wheels - Battery mounted high on frame - No arm — pusher bar only - 4-wheel drive, wide stance; - 4" diameter wheels - Battery mounted low and centered - Articulated arm with gripper - 4-wheel drive, narrower frame. TEACHER'S MODELED COMPARISON PARAGRAPH Both robots use four-wheel drive, which tells us both teams prioritized reliable traction and stability over simplicity. However, their wheel size choices reveal very different priorities: Robot A's 6-inch wheels give it a higher top speed — suggesting this team valued crossing the field quickly. Robot B's smaller 4-inch wheels sacrifice speed for more torque, better suited to climbing ramps or pushing against resistance. The most significant structural difference is battery placement. Robot A mounts the battery high, raising the center of gravity — a risk for a fast robot making sharp turns. This suggests the team either didn't anticipate tipping or accepted the risk to keep the frame simple. Robot B mounts the battery low and centered, a deliberate stability choice showing awareness of the physics involved. Finally, Robot A has no arm — just a pusher bar — while Robot B has a full articulated gripper. Robot A was designed for a strategy that doesn't require precise manipulation, while Robot B was built for a more complex task. The trade-off is that Robot B is heavier, slower, and harder to repair under time pressure. What made that effective: named specific features ("6-inch wheels" not "big wheels") · explained what each difference reveals about priorities · named a similarity first · identified the trade-off of each choice.

YOU DO Part 2B — Comparison Paragraph (Intermediate & Proficient)

Compare the two robot photos your teacher displays. Identify at least 2 similarities and 2 differences. For each difference, explain what it reveals about what each team prioritized. Use specific feature names and engineering vocabulary.

I DO Watch the teacher model an improvement proposal with engineering rationale

An improvement proposal isn't a complaint — it's a structured engineering argument: Current design → Problem → Proposed change → Trade-off. Every section must be specific. TEACHER'S MODELED IMPROVEMENT PROPOSAL — targeting Robot A's battery placement Current design; Robot A mounts the battery in a bracket near the top of the frame, approximately 10 inches above the ground plane.. Problem identified; A high center of gravity increases the risk of tipping during fast turns or when the robot contacts a field element. At 6-inch wheel diameter and high drive speed, lateral forces during turning are significant. A tipped robot cannot recover autonomously.. My proposed change; Relocate the battery to a low-mounted tray centered between the two drive rails, 2–3 inches above the ground plane. Use a REV battery strap and quick-release bracket so it can be swapped rapidly between matches.. Trade-off of my change; Low battery placement reduces interior frame space, which could interfere with wiring runs or a future mechanism. The team should plan cable routing before finalizing the tray position.. Notice: each section is specific — dimensions, component names, engineering reasons. No vague language like "it would be better if..."

YOU DO Part 2C — Improvement Proposal (Proficient)

Choose one design decision from the robot photo that you think could be improved. Follow the four-part structure below. Be specific — name dimensions, component types, and engineering reasons.

| Section | Your response |
| --- | --- |
| Current design |  |
| Problem identified |  |
| My proposed change |  |
| Trade-off of my change |  |

PART 3  ·  ENGINEERING NOTEBOOK SETUP (15 MIN)

## Your First Notebook Entry

Your engineering notebook is a record of your design process. In FTC competitions, judges read notebooks to understand how your team thinks. Every entry must be dated and signed. Every sketch must be labeled. Every decision must have a reason.

Notebook setup checklist:

- Title page with team name, year, and member names

- Table of contents page (leave 2 pages blank)

- First entry dated and signed by all members

- Definition of "intentional design" in your own words

- One design decision from the robot photo, with rationale

- Page number written at the bottom of the entry page

NOTEBOOK ENTRY Write today's full date at the top right. Then write: "Today I learned that intentional design means..." — finish this sentence in at least 3 sentences. Then sketch or describe one design choice from the robot photo and explain the engineering reason behind it.

PART 4  ·  EXIT REFLECTION (10 MIN)

## Checking Your Understanding

1. Write one sentence that defines intentional design without using the word "intentional."

2. What is one question you still have after today's class?

3. Rate your confidence on each objective (circle: Not yet / Getting there / Got it)

| Objective | Not yet | Getting there | Got it |
| --- | --- | --- | --- |
| I can define intentional design | ○ | ○ | ○ |
| I can identify design decisions in a robot photo | ○ | ○ | ○ |
| I set up my engineering notebook correctly | ○ | ○ | ○ |

EXTENSION / EXPLORATION Research a real-world engineering failure — good starting points include the Tacoma Narrows Bridge (1940), the Gimli Glider (1983), or the Therac-25 radiation machine (1985–1987). Write a 1-page analysis: What was the design intent? What constraint was underestimated or ignored? What would intentional design have caught?