INTRODUCTORY ROBOTICS · UNIT 1: ENGINEERING FOUNDATIONS # REV Servo Claw Design Challenge Class 4 of 5 · 74 minutes

| NAME _______________________________ | TEAM _______________________________ | DATE _______________________________ | PERIOD _______________________________ |
| --- | --- | --- | --- |

LEARNING OBJECTIVES 1. Complete the servo arm experiment and write a rule about arm length and grip force. 2. Complete the linkage experiment and compare single-pivot vs 4-bar mechanisms. 3. Produce three orthographic claw design sketches with labeled dimensions and a parts list. 4. Evaluate your three designs using a weighted decision matrix and document a design rationale.

Quick Experiment A  ·  Servo Arm & Force (15 min)

## How Does Arm Length Change What a Servo Can Lift?

Materials (per group)

1 REV Smart Robot Servo connected to a Control Hub · 1 popsicle stick or cardboard strip · tape · small zip-lock bag · coins · ruler

REV Smart Robot Servo — Key Specifications

Specification; Value. Range of motion; 0° – 270°. Stall torque; 13.5 kg·cm. No-load speed; 0.14 sec / 60°. Control signal; PWM via Control Hub.

Procedure:

Step 1 — Short arm (3 cm). Tape the stick to the servo horn so it extends 3 cm. Add coins to the bag until the servo stalls. Count and record.

Step 2 — Medium arm (6 cm). Reposition to 6 cm and repeat.

Step 3 — Long arm (9 cm). Reposition to 9 cm and repeat.

| Arm length | Approx. mass held before stall | More or less than 3 cm arm? |
| --- | --- | --- |
| 3 cm |  |  |
| 6 cm |  |  |
| 9 cm |  |  |
| 12 cm (predict) | Prediction: ___ |  |

1.  Write a rule in your own words: as the arm gets longer, the force at the tip gets...

As the arm got longer, the servo could hold... A shorter arm gives you... while a longer arm gives you...

2.  Your claw fingers aren't gripping hard enough. Based on your experiment, what is the one change you could make to finger length — and what would you give up?

To get more gripping force I would... The trade-off is that the fingers would then...

Quick Experiment B  ·  Linkage Comparison (10 min)

## How Does a Linkage Transfer Motion?

Materials (per group)

4 cardboard strips (~15 cm each) · 4 brass paper fasteners · 1 single cardboard strip for comparison

Procedure:

Step 1. Connect two strips with one fastener (single pivot). Hold one still and rotate the other. Sketch what you see.

Step 2. Connect all four strips in a loop (4-bar linkage). Hold one side still. Push one adjacent side. Observe the opposite side.

| Single pivot — what the opposite end does | 4-bar linkage — what the opposite end does |
| --- | --- |

3.  Which linkage keeps both ends more parallel throughout the motion? Why does that matter for a claw gripping a cube?

The ___ keeps the ends more parallel because... This matters for a claw because...

Part 3  ·  Ideate — Three Claw Designs (25 min)

## Sketch Three Different Mechanisms

Your claw must use exactly one REV Smart Robot Servo. You may use any REV structural components. Produce three genuinely different mechanism concepts — direct-drive, single-pivot finger, or 4-bar parallel jaw. Each sketch needs: front view + side view, labeled dimensions, servo and mount location marked.

| NOVICE Three different claw shapes (e.g. wide vs narrow fingers). Label at least 3 dimensions per sketch. | INTERMEDIATE Three different mechanism types. Both views with labeled dimensions. Mark servo and mounting interface on each. | PROFICIENT Three mechanism types. Full orthographic views. Show the target object (dashed 10 cm cube). Draw arrows showing how the servo's rotation causes each finger to move — start at the servo horn, end at the fingertip. |
| --- | --- | --- |

| Design A | Design B | Design C |
| --- | --- | --- |

Part 4  ·  Weighted Decision Matrix (12 min)

## Evaluate Your Three Designs

Set your weights BEFORE you rate any design. A weighted matrix multiplies each rating by the criterion's importance so the most critical factors count more.

Set your weights here first — before rating anything

Criterion; Weight (1–3); Justification (brief). Grip reliability on a 10 cm foam cube. Interface compatibility with REV extrusion frame. Build complexity (simpler = better under time pressure). Repairability during competition. Your own criterion:.

Now rate each design (1–3) and calculate weighted scores:

| Criterion | Wt | Design A rating × wt | Design B rating × wt | Design C rating × wt |  |
| --- | --- | --- | --- | --- | --- |
| Grip reliability |  |  |  |  |  |
| Interface compatibility |  |  |  |  |  |
| Build complexity |  |  |  |  |  |
| Repairability |  |  |  |  |  |
| Your criterion |  |  |  |  |  |
| WEIGHTED TOTAL |  |  |  |  |  |

Selected design: ___    Weighted score: ___

Design rationale (3–4 sentences — reference your highest-weighted criterion and name the trade-off you are accepting):

I chose Design ___ because my highest-weighted criterion was... The trade-off I am accepting is... I would change... if I had a fourth design option.

Part 5  ·  Exit Reflection (8 min)

## Checking Your Understanding

1.  Explain in one sentence what the stall torque spec on a servo tells you — using what you discovered in Experiment A.

2.  Rate your confidence:

| Objective | Not yet | Getting there | Got it |
| --- | --- | --- | --- |
| I can explain how arm length affects gripping force using my experiment data | ○ | ○ | ○ |
| I can describe the difference between a single-pivot and 4-bar mechanism | ○ | ○ | ○ |
| I produced three claw sketches with labeled orthographic views | ○ | ○ | ○ |
| My weighted matrix weights were set before I rated any design | ○ | ○ | ○ |

Extension / Exploration — Tolerance Everyone in your group cuts a strip that is exactly 10 cm long — freehand, no ruler. Measure each strip. In your notebook, write: "For my claw to grip reliably, the finger length needs to be [target] ± [acceptable range] cm, otherwise it will fail because [reason]." Try closing your fingers on the foam cube and measure the actual gap. The difference between what you drew and what you built is called manufacturing variation.

Introductory Robotics  ·  Unit 1, Class 4  ·  REV Servo Claw Design Challenge  ·  Page ____ of ____